DZ-11 documentation for implementers.

the dz-11a driver in its current release has been working fine for several

days now on our installation.

since we got the hardware last week, it has no proven field record yet.

the current copy should be considered as a pre-release one, and

the author will be grateful for all comments, reports, fixes and

suggestions.

if you do plan to use not all 8 lines, you'll save allocated

tty-structure space by reducing the number 'ndz11'.

for 16 lines, the driver must be modified. suggestions on how to do so

are in the header of the code. watch out for the implied restriction

that minor device number equals hardware line number.

for a dz-11c (current loop) it is probably wise to leave the routine

'dztimer' out. it's function is to watch carrier transitions, since they

do not generate interrupts on a dz-11, but a current loop connection has

no carrier at all. also leave out it's initial call from 'dzopen'.

note that the speed setting codes correspond to the dh-11 ones, as

described in stty (ii). speeds supported by the dh-11 and not by the dz-11

are defaulted to 300 baud.

speeds supported by the dz-11 and not by the dh-11 are added as codes

16, 17 and 18. see table 'dzspeedtab' in the driver.

flaws may exist in the two-stop bit and in the 6/7/8 data bit handling area.

nijmegen, november 19, 1976

author's address:

h.j. thomassen

computer graphics group

faculty of science, toernooiveld

university of nijmegen,

nijmegen, the netherlands.